BARLOW POND SOUTH

The fuel models on parcel F06 are low risk timbered fuels and low risk shrubs that make up marsh surrounding open water.

Parcel F28 is 50% low risk timber fuels and 50% moderate shrub fuels with no structures adjoining the shrub fuels which reduces wildfire risk to adjacent properties.

The E04 parcel consists of a steep ravine with a Southeast aspect. The toe of this slope is at the beach on Block Island Sound. Above the beach to the top of the bluff at the head of the ravine are continuous shrub fuels including both Maritime Shrub/Scrub and Successional Shrub forest. These shrub models are rated as High risk. The NE corner of this parcel, at the top of the slope, is a low risk timbered tree plantation. A small structure (an art studio) is perched on the edge of this ravine at the top of this steep SE aspect and is embedded in dense shrub fuels. The landowner should decide if some clearing to create a fire break around the structure is merited, or, since this is not a very expensive structure just accept risk of loss due to a possible wildfire. As mentioned before in this report fire behavior predicted in these shrub fuels may be over-predicted by the model due to reluctance of the Maritime Shrub / Scrub vegetation to burn unless under rare and extreme conditions. The large mansion on the main property is not at risk from fire. Further details on E04 are spelled out below.