Barlow Pond North, Island Pond and Clay Point Road

The fuel models on parcel F08 are low risk timbered and grass fuels.

F29 has an isolated patch of phragmites that is no risk due to location while most of the parcel is non-burnable.

F18 / F55 / F35a / F57 / E10 all are mixed woodlands with low risk timber and low risk shrub fuels.

E06 is mostly low risk timber fuels with a small patch of moderate risk shrub, but there are no private exposures of concern. A portion of E06 is an area of TL4 fuels (Small Downed Logs) that is an area where mature trees we felled to improve vista clearing for a potential home site nearby. The fuels are litter and small branches / tops of trees felled, not the large tree trunks on the ground which are resistant to burning as 1000 hour / 10,000 hour fuels.

E05 is a combination of low risk timber and low risk grassland.

F52 and F16a are low risk timber and patches of low risk grasses.

F16b is mostly low risk grasses with band of shrubs adjacent to non-burnable beach and marsh.

F21a & F21b are both timbered with moderate shrub understory.

The Middle Farms North parcels are timbered with interior marsh and open water that are surrounded by low risk shrub fuels.

F32 has a 2.5 acre field of heavy grass that can be kept as low risk with routine prescribed burning or mowing.

F02 is mostly the island in the Island Pond and although it has some shrubby fuels there is no risk of wildfire escaping the island.