EAST END BY BIG CLUB AND MUD POND

Parcel F13 fuels consist of a marsh overrun with phragmites (High risk grass fuel) adjacent to a low risk timbered woodland to the south of the marsh. The marsh is in a depression below adjacent properties to the south, but there is no risk of wildfire from the marsh affecting the adjacent homes as long as those landowners continue to maintain mowed fuel breaks. The woodland south of the marsh borders an open fairway on the golf course so there is no wildfire risk present there

Parcel F51 is low risk timbered fuels and low risk timbered shrub.

Parcel F50 is directly behind the grounds keeping maintenance yard of the Fishers Island Golf Club. It is low risk timber understory fuel model throughout most of the block, however the northern edge is a non-burnable shrub swamp that fringes open water to the north.

Parcel F48 is primarily low risk timber understory fuel model curving around a small coastal pond fringed by a non-burning shrub swamp in the center of the unit.  The eastern edge of the unit, east of the shrub swamp, is a thicket of high fire risk heavy brush (SH6) fuel model. The overall wildfire rating of Parcel F48 is moderate based on the fact that the brush to the east is bordered by non-burnable swamp and a road. Note this coastal pond (and its surrounding marsh) has variable water levels, so under persistent drought the swamp could potentially burn.

Parcel F47 and Parcel F56 are both timbered low wildfire risk shrub fuel models and easily accessible from road.