MIDDLE FARMS, TREASURE POND AND BARRIER BEACH

The Middle Farms Flats parcels (F19/F20 /F35b/E01/E03) consist of grass fuels including grass shrub fuels in the grassland itself. These fuels are low risk when they are burned or mowed. East of the grassland the parcels have low risk timber fuels.  West and south of the grassland is a mixture of low risk timber, moderate risk shrub fuels and high risk phragmites that basically hug shorelines of Island Pond. Since these fuels are placed between open water and the low risk maintained field they present low wildfire risk.

Barrier Beach parcels (F38/F38b) have both abundant Maritime Shrub/Scrub and phragmites grasses but are surrounded by water. F03 fuels are limited to a band of high risk Maritime Shrub between the waters of Island Pond and Block Island Sound. This band of Maritime Shrub continues east of the property line of Parcel F03 onto adjacent private lands. There is no history of these Maritime Shrub/Scrub habitats burning on Fishers Island, however these fuels may have a very long fire return interval (200 or more years).

The parcels north and south of Middle Farms Pond (north are F63/Town Owned/F17/F10/F26; south are F11/F09/F24/F23/F05) are generally low risk timber models with some low risk shrub areas and non-burnable wet marsh habitats. F26 has a band of maritime shrub surrounded by low risk fuels. Along the southern edge of F23 there is a maritime bluff band of fuels that runs east-west. This feature contains a good mix of shrub/scrub plants that will burn intensely under very rare conditions. The good news is that this band is hemmed in by low risk timber fuels, unburnable marsh thicket and non-burnable rock beaches. So the wildfire risk is low.